| K900 |  |  |
| --- | --- | --- |
| X86 Fanless Industrial Computer |  |  |
|  |  | Key Features - Design for Industrial Automation M2M application - Wide operating temperature range of -10°C~+60°C - Aluminum Chassis with fanless design - Support 6-36V Wide Voltage Input |
| System Parameters |  |  |
| Processor&Chipset | - •Onboard Intel® J1900 Four Cores, 2.0GHz, 2M Cache ,NM70 Chipset - •Onboard Intel® N100 Four Cores, 3.4GHz, 6M Cache ,Intel® UHD Graphics - •Onboard Intel® i3,i5,i7, 4th~14th Generation |  |
| Memory | 1 x DDR3 /DDR4 SO-DIMM socket, 2GB/4GB/8GB/16GB |  |
| Storage | 1 x 2.5" SATA 500GB/1TB/2TB 1 x Msata 16GB/32GB/64GB/128GB/256GB |  |
| Cooling System | Fanless design |  |
| BIOS | AMI 16MB DPI Flash |  |
| Watchdog Timer | 255 level, 0~255 sec |  |
| LAN | 1 x Realtek RT8111E, 10/100/1000M, Support PXE, Wake on Lan |  |
| Audio | Realtek ACL662 Audio controller, Line-out, Mic-in. |  |
| Expansion Port | 2 x MINI PCI-E full slot, 1 x SIM |  |
| OS | Linux/ Win dows XP/7/8/10/11 |  |
| Expansion Function |  |  |
| GPIO | Optional |  |
| 3G/4G | Optional |  |
| WIFI/BT | Optional(802.11 a/b/g/n, Bluetooth 4.0) |  |
| RS422/485 | Optional |  |
| IO Interface |  |  |
| COM | 2 x COM(Default RS232, 10x RS232 maximum) |  |
| RS422/485 | Optional (1x RS485/422) |  |
| VGA | 1x VGA with definition up to 1920x1200@60Hz, DB15 interface |  |
| HD MI | 1x HDMI, with definition up to 1920x1200@60Hz |  |
| USB | 3 x USB2.0+1 x USB3.0 |  |
| LAN | 1 x RJ45, 10/100/1000M(2RJ45 Optional) |  |
| Audio | Line-out, Mic-in |  |
| LPT | Optional |  |
| ATN | 2 x Antenna port |  |
| GPIO | Optional |  |
| Power Supply |  |  |
| Power Supply | 12V DC Power Input |  |
| Power Comsuption | <60W |  |
| Wide Voltage | Optional (DC 6~36V Wide Voltage, IPTS ) |  |
| Environments |  |  |
| Operating Temp | -10℃~50℃ |  |
| Storage Temp | -30℃~70℃ |  |
| Relative Humidity | 5~95%@10℃ (Non-condensing) |  |
| Vibration | 5~500Hz,1.5G,0.15mm peak value |  |
| Shake | 1.5 Grms, IEC 60068-2-64, 5 ~ 500 Hz, 1 hr/axis |  |
| Shock | 10 Grems, IEC 60068-2-64, Half sine wave,for 11 ms |  |
| EMC | CE/FCC Class A |  |
| Enclosure |  |  |
| Housing Material | All Aluminum Chassis with fanless design |  |
| Dimensions | 209 mm (W) x 179 mm (D) x 56.5 mm (H) |  |
| Net Weight | 2KG |  |